EE 332 Digital Communication

Project

Shreyash S Bharati

220102094

Flow Diagram :

Techniques Used :

1] Obtaining the symbol rate : Multiplied the signal r(nTs) with its conjugate r*(nTs) and using FFT sink obtained peaks at intervals of 1/T from centre frequency (0Hz).

2] Resampling : As I was getting fractional sample per symbols, I had to resample my data to a new frequency.

3] Obtaining constellation scheme : Used Costa’s Loop for carrier recovery while checking for the sample with minimum ISI.

4] Obtaining frequency offset : Exponentiated the signal according to the constellation scheme and took the FFT to observe shift in the original spectrum and the exponentiated spectrum.

Calculations :

The peaks were obtained at 0.8MHz and hence the symbol rate=0.8MHz. Here, sampling frequency Fs=4.096MHz.

Samples per symbol=4.096/0.8=5.12 which is fractional.

So, lowering Fs to 4MHz gave 5 samples per symbol.

Skip head=2                                 Skip head=3

Skip head=5                                       Skip head=1

Then, using Costa’s Loop the constellation scheme was obtained as 8-QAM.

Now, keeping the exponent value as 8, the frequency shift was obtained as 0.8-0.7996=0.0004MHz=400Hz.

Hence, q∆f=400 and q=8. Therefore, ∆f=50Hz.

Results:

| Symbol Rate | 0.8MHz=800kHz |
| --- | --- |
| Samples per symbol | 5 |
| Constellation Scheme | 8-QAM |
| Frequency Offset | 50Hz |